# L75ms – Avatar Configuration

# Version

| Version | Date | By | Description |
| --- | --- | --- | --- |
| 1.0 | 23/02/2021 | Chris Mann | Covering document for L75ms – Avatar Configuration package |

Contents

Version	2

Overview	3

Avatar Configuration	3

ARCAM.SerialNumber_v2_0_1	4

# Overview

This document gives an overview of the contents of the L75ms – Avatar Configuration package and should be read in conjunction with the other documentation contained in the package.

The package contains information and instructions on how to configure and Avatar module as part of the production process.  It is split into 2 sections each in their own directory:

## Avatar Configuration

This contains instructions and tools on how the Avatar should be configured during production. To summarise, the process is as follows:

- Load the Avatar Bootloader using an AMLogic USB Burning Tool

- Install the production version of the Avatar application layer code using a USB stick.

- Setup Google cast certificates, Serial Number, MAC Address and Region using the Serial Number Burner tool

- When all configuration is complete and verified, lock the device using Serial Number Burner tool.

| Document / Folder | Description |
| --- | --- |
| SA30 OEM programming procedure rev 0.2.pdf | This document describes in detail the steps outlined above which were used for the SA30 and which sjould be applicable to the L75ms. |
| ARCAM Avatar Initial Configuration v0_2.pdf | This document describes the Avatar commands that are used to configure the device. NOTE: There commands are built into the Serial Number Burner and should not be required to be run manually as part of the configuration process. |
| WinSCP_PuTTY_USB_burner | This folder contains installers for the USB Burning tool, PuTTY, and WinSCP. PuTTY and WinSCP can be used to run Avatar commands and access the Avatar file system before locking. Note: Once locked the Avatar file system cannot be accessed using the tools above. THE DEVICE SHOULD ONLY BE LOCKED ONCE THE CONFIGURATION HAS BEEN VERIFIED. |
| WinSCP_PuTTY_USB_burner/arcam_debug_key.ppk | The arcam_debug_key.ppk file is required by the Avatar when accessing it using either PuTTY or WinSCP. It is present to provide a level of secure access to the Avatar before it is locked. THIS SHOULD NEVER BE SHARED WITHOUT AUTOHORISATION FROM HARMAN. |

## ARCAM.SerialNumber_v2_0_1

This contains the current Serial Number Burner software which is used to install Google Cast Certificates, Serial Numbers and MAC Addresses.

| File / Folder | Description |
| --- | --- |
| ARCAM.SerialNumber.Burner.exe | This is the Serial Number Burner software referred to in the documentation |
| Exports | Contains example Serial Number DAT files that can be used to test the software (NOTE: These contain real Serial Numbers used for other products and should NOT be used in a production environment.) |
| Documentation | Contains the Serial Number Burner manual. |