# L75ms - SA30 Software Overview

# Version

| Version | Date | By | Description |
| --- | --- | --- | --- |
| 1.0 | 11/01/2021 | Chris Mann | Following the introductory meeting on 2021/01/11 this document was created to provide an overview of the architecture of the SA30 |
| 2.0 | 15/01/2021 | Chris Mann | Addition of section describing project setup. |

Contents

Version	1

Overview	2

System Components	2

Avatar Module	3

HDMI/ARC module	3

System Architecture	4

Application layers	4

Input Processing	5

UI and Menus	6

Upgrade	6

PIC Utilities versions	6

# Overview

This document provides a brief overview of the SA30 software architecture.  It describes the components used in the architecture, how they hang together, and a description of way inputs are processed.

It also contains a brief discussion on the differences between the different versions of the PIC Utilities framework library.

It is designed to be viewed alongside the code supplied in the L75ms – SA30 Software Pack

# System Components

The SA30 system architecture is built from a collection of components controlled by the PIC32MZ processor.

These cover all the areas of the SA30’s functionality, including:

- Avatar – This is used to co-ordinate access to streaming functionality, and also manages DIRAC room correction.

- HDMI/ARC – Provides the interface to HDMI ARC and eARC enabled TV’s.

## Avatar Module

The Avatar module is used to co-ordinate access to streaming functionality.  In addition to this it also:

- Manages DIRAC Room correction.

- Implements the MQA codec.

- Provides device discovery functionality for Control4 / Crestron

- Handles the devices TCP/IP Interface.

- Manages device updates (including OTA, USB, and web page update initiation)

Communication with the Avatar is over I2C.  Note that the Avatar module is the master device in all cases. The host is the slave.

During device start up a handshaking process is performed between the Avatar and the Host which can take up to 40 seconds during which time the device functionality is unavailable.

A description of the messaging protocol for messages transferred between the Host and the Avatar can be found in the document “ARCAM Avatar Messaging v0_51.pdf”

## HDMI/ARC module

The HDMI ARC module provides ARC, eARC and CEC functionality for the control of the SA30 by ARC enabled TV’s.

Communication between the Host and the HDMI ARC is done over RS232.

A description of the messaging protocol for messages transferred between the Host and the HDMI ARC module can be found in the document “ARCAM HDMI ARC Messaging v0_9.pdf”

# System Architecture

## Application layers

The SA30 code is split into a number of layers moving from application specific code, through shared framework code to low level functions.  The diagram below shows the dependency of the different layers on the layers below them.

| Layer | Description |
| --- | --- |
| Application | Contains code specific to the current device. This includes functions managing state and the user interface. |
| Framework (PIC Utilities) | A collection of modules designed to work with a single component or collection of related components. These modules are shared across multiple device implementations. Examples: - UART - Remote - ADC - Avatar |
| Microchip Harmony | A framework provided by Microchip that provides a common interface to peripherals. Examples: - UART - I2C - SPI |
| PLIB | Microchip supplied low level functions used for accessing and managing ports. |

## Input Processing

Internal communication and processing is managed using standard structured messages.

Standard inputs such as Buttons, Encoder and the Remote have any changes stored in an input context which holds the current state of the input.  This is regularly parsed and converted into a structured messaged which is processed through a set of common application level functions designed to handle messages.

Other inputs such as TCP/IP, RS232 and the Avatar generate a structured message as part of their input parsing, this again is processed through the same set of common application level functions

This approach has the advantage of providing a single point of processing for any message type irrespective of where the trigger for the processing was generated.

## UI and Menus

The framework contains a module for handling the display of menus.  It allows menus to be defined as a hierarchy with each menu identifying the functions required to integrate with the menu system (see app_menu.c)

## Upgrade

Upgrades are controlled by the Avatar module.  An Avatar release contains firmware files required for upgrading the Host micro and any other micros in the device (In the SA30 this is the Host Micro plus the HDMI/ARC module).

When an upgrade is initiated (either OTA, from a USB stick, or Web Page) the avatar first upgrades itself, then passes the device firmware upgrade file to the device for processing.  The firmware upgrade file must contain any individual firmware updates.  It is the devices responsibility to unpack and distribute the firmware updates to the relevant components.  A file format for the firmware upgrade file is defined in ARCAM Avatar Update v0_4.pdf

## PIC Utilities versions

A development version of PIC Utilities v3 is included in the Code folder.   It begins to implement the changes in approach between version 2.1 and version 3 but is not yet a full implementation.

PIC Utilities divides functionality into modules e.g. I2C, RS232, Avatar.  Each module has one or more header and source files depending on its complexity.

Between version 2.1 and version 3 changes have been made in the way functionality is split between source and header files, these include:

- A hardware abstraction layer (see spdif_rx) that can be configured either at design or run-time.

- An events system allowing events to be raised and processed in the application layer, or modules that depend on the current module. (see spdif_rx, adc)

- The introduction of reusable application level code specific to the module (menu functions, settings etc...)

Note: As mentioned this is still under development.  Full v3 modules are put into sub-directories under the main utilities directory, the other will be converted over the next few months as required.

# System Setup

The L75ms – SA30 Software Pack contains a code directory containing the code written specifically by Arcam for the SA30.

To run the code you will need the following:

- Create the project directory hierarchy.

- Install the MPLAB X IDE, XC32 Compiler, Harmony and PIC Legacy libraries

## Project Directory structure

The project code relies on a relative file structure, so before starting you need to decide on a base directory to hold the code.

Assuming the base directory is D: the project structure will look like this:

- D:

  - microchip

      - harmony

        - v2_06

  - Projects

    - FileSystemBootloader

    - PICUtilities

      - v2_1

      - v3

    - SA30-SA750

## Software Installation

### Microchip

The Microchip software can be downloaded from here…

#### MPLAB X IDE

We are using MPLAB X v5.00 available from here…

https://www.microchip.com/development-tools/pic-and-dspic-downloads-archive

#### Microchip Harmony

We use version 2.06.  Note this should be installed in the correct location in the project structure (see above)

https://www.microchip.com/en-us/development-tools-tools-and-software/embedded-software-center/mplab-harmony/mplab-harmony-v2

#### Microchip XC32 Compiler

We use version 2.10

https://www.microchip.com/development-tools/pic-and-dspic-downloads-archive

#### PIC32 legacy Peripherals library

https://www.microchip.com/en-us/development-tools-tools-and-software/mplab-xc-compilers

### SA30

The three SA30 code directories should be copied from the Software pack code directory into the Projects directory (FileSystemBootloader, SA30-SA750, PICUtilities).

The Code folder contains a microchip directory.  This contains specific modifications that we have made to the Harmony libraries, and should be copied over the Harmony v2_06 directory.